ELECTRONICS PRACTICALS
CIRCUIT DIAGRAMS

Simple wiring diagrams for last-minute exam revision

These are simplified wiring diagrams for the five practicals in your study guide. If your class notebook gives different pin numbers, follow your notebook.

# Practical 1 — LED blinking

## Connections to remember

GPIO 2 → 220 Ω resistor → LED anode (+).

LED cathode (−) → GND.

# Practical 2 — LED webpage control

## Connections to remember

LED hardware is the same as Practical 1.

Phone/laptop and ESP32 communicate over the same Wi-Fi network.

# Practical 3 — DHT11 sensor

## Connections to remember

VCC → 3.3 V; DATA → GPIO 4; GND → GND.

Check the printed sensor labels; module pin order can vary.

# Practical 4 — I²C LCD

## Connections to remember

SDA → GPIO 21; SCL → GPIO 22; GND → GND.

Check LCD supply voltage and ensure I²C lines are safe for 3.3-V ESP32 GPIO.

# Practical 5 — DC motor + L298N

## Connections to remember

GPIO 26 → IN1; GPIO 27 → IN2.

Motor → OUT1/OUT2; external motor supply → L298N motor supply terminals.

Common ground is required; ensure ENA is enabled and disconnect power before changing wires.

# Quick pin revision

| Practical | ESP32 pins | Connect to |
| --- | --- | --- |
| 1. LED blink | GPIO 2 | Resistor + LED; cathode to GND |
| 2. LED webpage | GPIO 2 | Same LED circuit; Wi-Fi control |
| 3. DHT11 | GPIO 4 | Sensor DATA; VCC 3.3 V; GND |
| 4. LCD I²C | GPIO 21, 22 | SDA, SCL; VCC and GND |
| 5. DC motor | GPIO 26, 27 | L298N IN1, IN2; motor on OUT1/OUT2 |

# Safety

Disconnect power before rewiring. Never connect a DC motor directly to an ESP32 GPIO. Use a suitable driver and motor power supply. Check module voltage requirements.