Electronics Practical: Object Detection Using ESP32-CAM

1. Aim

To detect and identify objects in real time using an ESP32-CAM and a TinyML model trained using Edge Impulse.

2. Apparatus Required

- ESP32-CAM development board (AI-Thinker)

- ESP32-CAM-MB USB baseboard

- RHYX M21-45 camera module

- Laptop or computer

- USB cable

- Wi-Fi network or hotspot

- Arduino IDE software

- Edge Impulse Studio

- Images of objects for training the model

Easy way to remember: Board, camera, laptop, USB cable, Wi-Fi and software.

3. Principle

This project works on the principle of TinyML (Tiny Machine Learning).

TinyML allows a machine-learning model to run on a small, low-power device such as the ESP32-CAM.

The camera captures images, and a trained model analyses them to identify the objects and their positions. The model is trained using labelled images in Edge Impulse and then uploaded to the ESP32-CAM.

After uploading the model, object detection can take place directly on the board without requiring a continuous internet connection.

Easy explanation: The camera captures an image, and the trained AI model identifies the object and shows where it is located.

4. Working

- Hardware connection: The ESP32-CAM is connected to a laptop using the USB baseboard.

- Image capturing: The camera captures images of different objects.

- Data collection: The captured images are uploaded to Edge Impulse Studio.

- Labelling: Boxes are drawn around the objects, and each object is given a name or class label.

- Model training: Edge Impulse uses the labelled images to train and test the object-detection model.

- Model deployment: The trained model is exported as a library and included in the ESP32-CAM program.

- Object detection: The ESP32-CAM captures new images and uses the trained model to identify objects and their positions.

- Displaying results: The detected objects and their locations are displayed on a webpage or in the Serial Monitor.

Easy way to remember the working:

Capture → Label → Train → Upload → Detect → Display

5. Uses / Applications

- Security systems: To detect objects in surveillance applications.

- Smart homes: To identify objects for home automation.

- Industrial automation: To detect and identify objects during automated processes.

- Monitoring systems: To observe objects in real time.

- Educational projects: To learn artificial intelligence, machine learning and computer vision.

- Embedded systems: To perform object detection using a low-cost, low-power device.